Digital Signal Processing ENGR 4333/5333

Sample Test 1

Date:			Time: 75 minutes				Name:

Q1) Find the energy of the signal x[n] shown in Fig. 1

Q2) Answer the following

- Plot the signal x[3 - n] where x[n] is shown in Fig. 1.

- Plot the signal y[n] = n (u[n] – u[n-5])

- The signal x(t) = cos(50πt) is sampled at a sampling rate of 200 samples/second. Find the discrete signal x[n].

- What is the digital frequency and the fundamental period of the signal x[n]=cos(0.3πn)

- Is the system y[n] = nx[n − 1] time invariant?

Q3) A TV signal has a bandwidth of 4 MHz. This signal is sampled, quantized, and binary coded.

- Determine the sampling rate if the signal is to be sampled at a rate 25% above the Nyquist rate.

- If the samples are quantized into 512 levels, determine the number of binary pulses required to encode each sample.

- Determine the binary pulse rate (bits/s) of the binary coded signal.

Q4) A signal x(t) is converted to a binary signal. If the signal-to-quantization-noise ratio (SQNR) is required to be at least 60 dB, determine the minimum number of quantization levels L required, if the power of the signal x(t) is 5 watts. Determine the actual SQNR obtained with this minimum L.

Q5) Find a difference equation, including initial conditions, that approximates the behavior of the second-order difference equation d2y(t)/dt2 + 3y(t) = dx(t)/dt, where y(0) = 0 and dy(0)/dt = 3. Take the sampling interval as T = 0.05.

Q6) Determine the zero-input response for the system specified by the difference equation below and initial conditions y[-1] = 1 and y[0] = 2

y[n+2] - y[n + 1] + 0.16y[n] = 4x[n+2]

Q7) Determine a closed-form representation of the unit impulse response h[n] for the system specified by the difference equation

y[n] - y[n − 1] + 0.25y[n − 2] = 4x[n]